Government of Karnataka Department of Technical Education Board of Technical Examinations

Preparatory Examination 01

Course: Basics of Digital Logic and Computer Organization	Course Code: 25CS11I Time: 3 Hrs	 Max Marks: 100

Instructions: For Part-A questions, only the first written answers will be considered for evaluation.

PART A

- Select the correct answer from the choices given  15x1 = 15

  - Which of the following is a binary number?

    - 1011	b) 1234	c) 8F	d) 29

Ans: a)1011

  - The truth table of a NOT gate has how many inputs?

    - 0	b) 1	c) 2	d) 3

Ans: b)1

  - ASCII stands for:

    - American Standard Code for Information Interchange

    - Automated System for Character Identification and Input

    - Analog Signal Code for Integrated Interfaces

    - Advanced Source Code for Internet Interaction

Ans: a)American standard code for information interchange

  - Which of the following is a universal gate?

    - NAND	b) OR	c) XOR	d) NOT

Ans: a) NAND

  - The output of a full-subtractor is:

    - Difference and borrow	b) Sum and carry	c) Only borrow	d) Only difference

Ans: a) Difference and borrow

  - A D flip-flop is also called:

    - Delay flip-flop	b) Double flip-flop	c) Down counter	d) Direct flip-flop

Ans: a) Delay flip-flop

  - The main memory communicates directly with the:

    - Monitor	b) Keyboard	c) CPU	d) Hard disk

Ans: c) CPU

  - The register which holds the address of the next instruction is:

    - IR	b) MAR	c) PC	d) MDR

Ans: C) PC

  - USB stands for:

    - Universal Serial Bus	b) United System Bus	c) Unique Serial Bridge	d) Universal Signal Base

Ans: a) Universal Serial Bus

  - Which of the following connects internal components of the computer?

    - Compiler	b) Bus	c) ROM	d) SSD

Ans: b) Bus

  - What is the difference between NAND and NOR gates?

    - NOR is faster

    - NAND inverts OR, NOR inverts AND

    - NAND inverts AND, NOR inverts OR

    - Both are same

Ans: C) NAND inverts AND, NOR inverts OR

  - A shift register moves data:

    - Randomly	b) Parallel only	c) Bit by bit	d) In analog form

Ans: C) Bit by bit

  - Why is binary number system used in digital systems?

    - It is easy to understand for humans

    - It reduces memory usage

    - It can be represented using ON and OFF states

    - It provides more colors in graphics

Ans: C) It provides more colors in graphics

  - You want to simulate a circuit that counts from 0 to 7 repeatedly. Which circuit should be used?

    - Half Adder	b) Synchronous Counter	c) Demultiplexer	d) Decoder

Ans: B) Synchronous Counter

  - You need to invert a signal before sending it to a microcontroller. Which logic gate is appropriate?

    - AND	b) OR	c) NOT	d) XOR

Ans: C) NOT

- Fill in the blanks by choosing appropriate answer from those given in the bracket: (one,output, current, real numbers, outputs)	5X1 = 05

  - Floating point representation is used to store real numbers

  - The truth table of a logic gate shows all possible combinations of inputs and their corresponding output.

  - A flip-flop stores one bit of data.

  - The Instruction Register (IR) holds the current instruction.

  - A printer is categorized as an output device.

PART B

- Answer any FIVE questions:	5X2 = 10

  - Convert the hexadecimal number 2A to its binary equivalent.

  - State the base value of each of the following number systems: binary, octal, decimal, and hexadecimal.

  - State any two Boolean laws used in circuit simplification.

  - List any two examples of sequential circuits.

  - What is the function of the Control Unit in a computer?

  - State the function of the address bus.

  - Name any two types of memory used in a typical computer.

  - List any two commonly used I/O ports.

PART C

VI. Answer any FIVE questions:	5X3 = 15 .

- Explain the need for data representation in digital systems.

- Differentiate between binary and BCD number systems with suitable examples.

- Describe how a NAND gate can be used to build an AND gate.

- Compare SOP and POS (Product of Sums) expressions with examples.

- What is a Multiplexer? List the applications of a multiplexer.

- Explain the Von Neumann architecture and list its key components.

- Compare the roles of data bus, address bus, and control bus in a computer system.

- Differentiate between primary, secondary, and tertiary memory with one example for each.

PART D (Section I)

- Answer any FIVE questions:	5X5 = 25 0

  - Describe the purpose of BCD (Binary-Coded Decimal) in digital systems. Explain how the decimal number 59 is represented in BCD and compare it with its pure binary representation.

  - Explain the function of basic logic gates (AND, OR, NOT) with their symbols, truth tables, and one practical use for each.

  - Explain the step-by-step procedure to derive a Boolean expression in POS (Product of Sums) form from a given truth table with three input variables A, B, and C with one output Y.

| A | B | C | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

  - Describe the applications of Multiplexer and De-multiplexer.

  - Explain the instruction execution cycle in a CPU. What happens during fetch, decode, and execute stages?

  - Explain any three addressing modes used in CPU instruction execution. Give an example

for each.

  - Explain memory hierarchy and how CPU interacts with different levels of memory during instruction execution.

  - Compare parallel and serial bus architectures. Explain one advantage and disadvantage of each with suitable examples.

PART D (Section II)

- Answer any THREE questions:	10X3 = 30

  - Given a logic circuit explain the procedure to construct its truth table. Apply this procedure to the given circuit diagram and present the complete truth table showing all possible input combinations and corresponding outputs.

Y

  - You’ve been given a mysterious circuit that encodes or decodes a secret binary message using XOR gates. Your task is to figure out the input, the circuit logic, and the hidden message. If the binary message M=11001and the secret key K=10101 what is the encoded message X To decode the message, the encoded message X is passed through the same XOR gate with the original key K=10101. What is the decoded message M.

  - You are designing the control system of a washing machine. The machine must go through a fixed sequence: Fill → Wash → Rinse → Spin → Stop. Each stage runs for a fixed amount of time and then automatically moves to the next.

    - Is this an example of a combinational or sequential circuit? Explain why.

    - Name any two digital components used to implement this sequence and describe their roles.

  - Explain the key CPU performance metrics such as clock speed, number of cores, and cache memory. Provide an example for each to show how it affects computer performance.

  - Differentiate between primary, secondary, and tertiary memory. Give two examples of each and their typical applications.